Code No. 5ES103ME

METHODIST COLLEGE OF ENGINEERING & TECHNOLOGY                          (An Autonomous Institution)

B.E. (ECE) I-Semester (Supplementary) Examination, September-2023

Subject: ELEMENTS OF MECHANICAL ENGINEERING

Time: 3 hours                                                                                            Max.Marks:60

Note: Missing data, if any, maybe suitably assumed.

PART-A

Answer All the questions.

| Q.No. | Questions | Marks | CO | BTL |
| --- | --- | --- | --- | --- |
| 1. a | What is a bio-fuel? Explain its advantages. | 2 | 1 | I |
| b | Distinguish between an open system and isolated system. | 2 | 1 | I |
| c | Define Heat Transfer. Mention its applications in Mechanical Engineering. | 2 | 2 | II |
| d | State Fourier’s law of conduction. | 2 | 2 | I |
| e | Define system and write different types of system. | 2 | 3 | I |
| f | What do you understand by slip in Belt drives? | 2 | 3 | IV |
| g | State the merits and demerits of Spot Welding. | 2 | 4 | II |
| h | Mention the applications of brazing. | 2 | 4 | I |
| i | Explain the advantages of die casting. | 2 | 5 | I |
| j | Enlist various types of Lathe machine. | 2 | 5 | II |

PTO

PART-B

Answer Any Five questions.

| Q.No. |  | Questions | Marks | CO | BTL |
| --- | --- | --- | --- | --- | --- |
| 2. | a | What do you understand by Global warming? How to control it? | 4 | 1 | I |
|  | b | Explain the concept of Thermodynamic equilibrium. | 4 | 1 | I |
| 3. |  | Thickness of the wall of an oven is 25 cm with thermal conductivity as 0.7 W/mK. The temperature interior of the oven is maintained at 400°C and that of outside the wall is 30°C. The total surface area of the wall of the oven is 4m2. Calculate the heat flow rate. | 8 | 2 | II |
| 4. |  | Deduce the equation for the length of a Cross belt drive. | 8 | 3 | III |
| 5. | a | Explain about the equipment used for Gas welding. | 4 | 4 | II |
| 6. | a | Describe the process of Hot Rolling with the help a neat sketch. | 4 | 5 | IV |
|  | b | Explain the working of a Lathe machine? | 4 | 5 | I |
| 7. |  | An ideal gas of 2 kg mass is adiabatically compressed from pressure 100 kPa and temperature 220K to a final pressure of 400 kPa. Determine (i) initial volume, (ii) final volume and temperature and (iii) work performed. Take cp= 1 kJ/kg K and cv = 0.707 kJ/kg K. | 8 | 2 | V |
| 8. | a | An open belt running over two pulleys of 120 mm and 300 mm diameters connects two parallel shafts at 2m apart and to transmit a power of 3 kW from smaller pulley which rotates at 312 rpm. Determine the length of the belt required. | 8 | 3 | III |
| 9. | a | Explain the similarities and dissimilarities between Heat and Work | 4 | 2 | VI |
|  | b | Illustrate the concept of Extrusion process. | 4 | 5 | II |

******