Code No. 5ES103ME

METHODIST COLLEGE OF ENGINEERING & TECHNOLOGY                          (An Autonomous Institution)

B.E. (ECE) I-Semester (AICTE) Examination, March-2023

Subject: ELEMENTS OF MECHANICAL ENGINEERING

Time: 3 hours                                                                                            Max.Marks:60

Note: Missing data, if any, maybe suitably assumed.

PART-A

Answer All the questions.

| Q.No. | Questions | Marks | CO | BTL |
| --- | --- | --- | --- | --- |
| 1. a | What are the applications of nuclear energy? | 2 | 1 | I |
| b | Define thermodynamic system. | 2 | 1 | I |
| c | State the Newton’s law of cooling. | 2 | 2 | I |
| d | Distinguish between parallel flow and counter flow heat exchangers | 2 | 2 | II |
| e | Mention the specific applications of V-belts. | 2 | 3 | IV |
| f | How gear drives are classified? | 2 | 3 | I |
| g | Explain Arc Welding. | 2 | 4 | I |
| h | Explain brazed joint and soldered joint? | 2 | 4 | II |
| i | Define system. | 2 | 5 | I |
| j | Enlist the accessories carried out on a Lathe Machine | 2 | 5 | I |

PART-B

Answer Any Five questions.

| Q.No. |  | Questions | Marks | CO | BTL |
| --- | --- | --- | --- | --- | --- |
| 2. | a | What do you understand by Renewable energy? Explain the various sources of Renewable energy. | 5 | 1 | I |
|  | b | Discuss the similarities between heat and work. | 3 | 1 | I |
| 3. |  | A plane wall of 110 mm thick with an area of 4 m2 has a steady surface temperatures of 140°C and 80°C as shown in figure below. Take thermal conductivity as k= 10.2 W/m°C and determine the rate of heat flow across the plane wall. t1=140°C | 8 | 2 | V |
|  |  | t2=80°C L=110mm |  |  |  |
| 4. | a | Sketch and explain various types of gear. | 8 | 3 | II |
| 5. | a | Discuss the Principle of gas welding with a neat sketch. | 4 | 4 | V |
|  | b | Explain the types of flames used in Gas welding. | 4 | 4 | VI |
| 6. | a | Explain sand casting process. | 5 | 5 | IV |
|  | b | Explain the working process of lathe machine. | 3 | 5 | II |
| 7. |  | 1.2 kg of Nitrogen contained in a cylinder at 6 bar pressure and at 310 K temperature expands three times to its original volume at constant pressure. Determine: (i) initial volume (ii) final temperature , (iii) Work done by gas and (iv) heat added. Take Cp for Nitrogen as 1.05kJ/kg K and R = 295 J/kgK | 8 | 1 | V |
| 8. |  | Deduce the expression for the length of an Open belt drive | 8 | 3 | III |
| 9. | a | Mention the advantages and applications of Seam welding. | 3 | 3 | I |
|  | b | Explain various operations on a Milling Machine. | 5 | 4 | II |

******