Code No.ES202EE

METHODIST COLLEGE OF ENGINEERING & TECHNOLOGY

(An Autonomous Institution)

B.E. (EEE) II-Semester (AICTE) Supplementary Examination, February-2023

Subject: PRINCIPLES OF ELECTRICAL & ELECTRONICS ENGINEERING

Time: 3 hours                                                                                       Max.Marks:60

Note: Missing data, if any, maybe suitably assumed.

PART-A

Answer All the questions.

| Q.No. | Questions | Marks | CO | BTL |
| --- | --- | --- | --- | --- |
| 1. a | Define Ohm’s law | 2 | 1 | L1 |
| b | Define dependent sources | 2 | 1 | L1 |
| c | Define loop, branch. | 2 | 2 | L1 |
| d | Obtain delta equivalent for the star circuit with resistors Ra = 3Ω, Rb = 2Ω, Rc = 1Ω in star | 2 | 2 | L2 |
| e | State maximum power transfer theorem | 2 | 3 | L2 |
| f | State super position theorem | 2 | 3 | L2 |
| g | What is forward bias and reverse bias in a PN junction | 2 | 4 | L1 |
| h | Give applications of zener diode | 2 | 4 | L3 |
| i | Why transistor is called as a current controlled device? | 2 | 5 | L1 |
| j | Why FET is called unipolar device | 2 | 5 | L1 |

PART-B

Answer Any Five questions.

| Q.No. |  | Questions | Marks | CO | BTL |
| --- | --- | --- | --- | --- | --- |
| 2. | a | Differentiate between active and passive elements | 4M | 1 | L4 |
|  | b | By using star-delta transformation for the following figure 1. Find the current ‘I’ supplied by the battery? | 4M | 1 | L3 |
| 3. |  | Find the equivalent resistance connected between Rab for the circuit shown in figure | 8M | 2 | L3 |
| 4. | a | State and explain Thevenin’s theorem | 4M | 3 | L2 |
|  | b | Using Thevinis theorem find the voltage V in the circuit shown in figure | 4M | 3 | L3 |
| 5. | a | Draw and explain the V-I characteristics of a pn junction | 4M | 4 | L3 |
|  | b | Compare Half wave rectifier, Full wave rectifier and Bridge rectifier in any four aspects. | 4M | 4 | L4 |
| 6. |  | Illustrate the input and output characteristics of BJT in three configurations. | 8M | 5 | L3 |
| 7. | a | Explain the Kirchhoff’s laws? | 3M | 1 | L1 |
|  | b | Find the current through 12Ω resistor using nodal analysis | 5M | 2 | L3 |
| 8. | a | State and explain reciprocity theorem | 4M | 3 | L1 |
|  | b | Find the rms value, average value and form factor of a half wave rectified voltage | 4M | 4 | L2 |
| 9. | a | Explain in detail the volt-ampere relationship of R, L and C elements with neat diagrams. | 4M | 1 | L2 |
|  | b | Explain the necessity of filter after the rectifier circuit | 4M | 4 | L1 |

******