Code No.

METHODIST COLLEGE OF ENGINEERING & TECHNOLOGY

(An Autonomous Institution)

M.E II-Semester (Supplementary) Examination, November-2023

Subject: AUTOMATION

Time: 3 hours                                                                                            Max.Marks:60

Note: Missing data, if any, maybe suitably assumed.

PART-A

Answer All the questions. (05X2M=10M)

| Q.No | Questions | Marks | CO | BTL |
| --- | --- | --- | --- | --- |
| 1 a | Differentiate between flexible automation and programmable automation. | 2 | 1 | 2 |
| b | List out the common reasons for downtime on an Automated Production line. | 2 | 2 | 1 |
| c | What is an industrial control system? | 2 | 3 | 1 |
| d | What is an automated guided vehicle system? | 2 | 4 | 1 |
| e | Mention different types of automated inspection. | 2 | 5 | 2 |

PART-B

Answer Any Five questions. (05X10M=50M)

| Q.No. |  | Questions | Marks | CO | BTL |
| --- | --- | --- | --- | --- | --- |
| 2 | a | Explain in detail about the reasons for automation. | 5 | 1 | 4 |
|  | b | Enumerate the differences between automation and mechanization. | 5 | 1 | 1 |
| 3 | a | A 30 station Transfer line is being proposed to machine a certain component currently produced by conventional methods. The proposal received from the machine tool builder states that the line will operate at a production rate of 100 pc / hr at 100% efficiency. From a similar transfer line it is estimated that breakdowns of all types will occur at a frequency of F = 0.20 breakdowns per cycle & that the average downtime per line stop will be 8.0 minutes. The starting blank that is machined on the line costs Rs. 5.00 per part. The line operates at a cost for 100 parts each & the average cost per tool = Rs. 20 per cutting edge. Compute the following: - Production rate - Line efficiency - Cost per unit piece produced on the line | 10 | 2 | 3 |
| 4 | a | What are Actuators? Discuss any one actuator in detail. | 5 | 3 | 2 |
|  | b | Differentiate between Continuous Control and Discrete Control. | 5 | 3 | 2 |
| 5 | a | Explain the following with respect to AGV. - Dead reckoning. - Vehicles acquisition distance. | 5 | 4 | 2 |
|  | b | What is AS/Rs ? Explain different AS/Rs. | 5 | 4 | 2 |
| 6 | a | Write a short note on automated inspection. | 5 | 5 | 2 |
|  | b | Explain different types and configurations of CMM. | 5 | 5 | 2 |
| 7 | a | Explain the various functions of automation information processing in manufacturing. | 5 | 1 | 2 |
|  | b | Discuss the various control functions of an automated transfer line. | 5 | 2 | 2 |
| 8 | a | Explain an adaptive control in detail? | 5 | 3 | 2 |
|  | b | Discuss various considerations in selection of robot for material handling applications. | 5 | 4 | 2 |
| 9 | a | Explain in detail about Machine Vision. | 10 | 5 | 2 |

******