Code No. PC304EC

METHODIST COLLEGE OF ENGINEERING & TECHNOLOGY                          (An Autonomous Institution)

B.E. (ECE) III-Semester (AICTE) (Supplementary) Examination, AUGUST -2023

Subject: SWITCHING THEORY AND LOGIC DESIGN

Time: 3 hours                                                                                            Max.Marks:60

Note: Missing data, if any, maybe suitably assumed.

PART-A

Answer All the questions.

| Q.No. | Questions | Marks | CO | BTL |
| --- | --- | --- | --- | --- |
| 1. a | State and prove Axioms of Demorgons theorem? | 2 | 1 | 2 |
| b | How do you find the Dual of an expression? | 2 | 1 | 1 |
| c | Simplify the given 4 variable Boolean using the karnaugh map. F(A, B, C, D) = ∑(0,3, 5, 7, 8,9,11, 10, 12, 15) | 2 | 2 | 2 |
| d | Convert the following SOP to POS Y=AB’+BC’+AB’C | 2 | 2 | 2 |
| e | Implement OR gate using Nand Gate | 2 | 3 | 3 |
| f | Define Truth table of Full Adder | 2 | 3 | 1 |
| g | Define Sequential circuit? Give an Example for it. | 2 | 4 | 1 |
| h | Distinguish between latch and Flip-flop | 2 | 4 | 2 |
| i | Differentiate PLA and PAL | 2 | 5 | 2 |
| j | Mention the Differences between Mealy and More Machine | 2 | 5 | 2 |

PTO

PART-B

Answer Any Five questions.

| Q.No. |  | Questions | Marks | CO | BTL |
| --- | --- | --- | --- | --- | --- |
| 2. | a | Implement the grey code number to equivalent binary code convertor ? | 5 | 1 | 3 |
|  | b | Convert (4F2B)16 to decimal, binary, octal. | 3 | 1 | 2 |
| 3. | a | Implement the Function f(a,b,c,d)= ∑ m {1,2,4,7,8,10,12,14} | 4 | 2 | 3 |
|  | b | Simplify the given function using tabular method (MC-Cluskey method). F (A, B, C, D) = ∑ m(0,1, 2, 3,5, 6, 7, 8, 10, 12, 13) | 4 | 2 | 3 |
| 4. | a | Why a NAND and NOR gates are known as universal gates? Simulate all the basic Gates. | 4 | 3 | 2 |
|  | b | Implement the function f (,) =Ʃ m(1,2, 3,4,5,7) using Nand gates only | 4 | 3 | 3 |
| 5. | a | Design a 4bit Binary code to Grey code convertor and Implement using Logic Gates | 4 | 4 | 3 |
|  | b | Convert JK Flip-Flop to SR Flip-Flop? and Implement | 4 | 4 | 4 |
| 6. | a | What are Prime Implicants? Define Essential Prime Implicants ? | 3 | 2 | 2 |
|  | b | Define POS and SOP forms with examples? | 5 | 1 | 2 |
| 7. | a | Implement Full Adder using 3-8 Decoder | 5 | 3 | 3 |
|  | b | Define Binary adder/Substractor ? | 3 | 3 | 2 |
| 8. | a | Design Mod-6 Ripple Counter? | 4 | 4 | 4 |
|  | b | with neat sketch Define the Operation of Bidirectional shift register | 4 | 4 | 2 |
| 9. | a | Design and explain the operation of Weighing machine | 4 | 5 | 3 |
|  | b | Define CPLDs in detail with Neat sketch? | 4 | 5 | 2 |

******