Code No. PC304EC

METHODIST COLLEGE OF ENGINEERING & TECHNOLOGY                          (An Autonomous Institution)

B.E. (ECE) III-Semester (AICTE) (Regular) Examination, Feb -2023

Subject: SWITCHING THEORY AND LOGIC DESIGN

Time: 3 hours                                                                                            Max.Marks:60

Note: Missing data, if any, maybe suitably assumed.

PART-A

Answer All the questions.

| Q.No. | Questions | Marks | CO | BTL |
| --- | --- | --- | --- | --- |
| 1. a | What are Axioms? and Define any Two? | 2 | 1 | 2 |
| b | How do you find the Dual of an expression? | 2 | 1 | 1 |
| c | Simplify the given 4 variable Boolean using the karnaugh map. F(A, B, C, D) = ∑(0, 2, 5, 7, 8,9,11, 10, 14, 15) | 2 | 2 | 2 |
| d | Convert the following SOP to POS Y=AB’+B’C+A’B’C | 2 | 2 | 2 |
| e | Implement OR Gate using NOR Gate | 2 | 3 | 3 |
| f | Define Truth table of Half Adder | 2 | 3 | 1 |
| g | Define Combinational circuit? Give an Example for it. | 2 | 4 | 1 |
| h | Distinguish between latch and Flip-flop | 2 | 4 | 2 |
| i | Differentiate PLA and PAL | 2 | 5 | 2 |
| j | Mention the Differences between Mealy and More Machine | 2 | 5 | 2 |

PTO

PART-B

Answer Any Five questions.

| Q.No. |  | Questions | Marks | CO | BTL |
| --- | --- | --- | --- | --- | --- |
| 2. | a | Perform subtraction operation 0011.1001 from 0010.1110 using 2's complement method . | 4 | 1 | 3 |
|  | b | Convert (0EBA)16 to decimal, binary, octal. | 4 | 1 | 2 |
| 3. | a | Implement logic for the Function F(a,b,c,d)= ∑ m (1,2,4,7,8,11,12,15) | 4 | 2 | 3 |
|  | b | Simplify the given function using tabular method (Quine-McCluskey method). F (A, B, C, D) = ∑ m(0, 2, 3, 6, 7, 8, 10, 12, 13) | 4 | 2 | 3 |
| 4. | a | Why a NAND and NOR gates are known as universal gates? Simulate all the basic Gates. | 4 | 3 | 2 |
|  | b | Implement the function f (,) =Ʃ m(1,2, 3,4,5, 6, 7) using NOR gates only | 4 | 3 | 3 |
| 5. | a | Design a 4bit Gray code to Binary code convertor and Implement using Logic Gates | 4 | 4 | 3 |
|  | b | Convert SR Flip-Flop to JK Flip-Flop? and Implement | 4 | 4 | 4 |
| 6. | a | What are Prime Implicatnts? Define | 3 | 2 | 2 |
|  | b | Define SOP and POS forms with examples? | 5 | 1 | 2 |
| 7. | a | Implement Full Adder using 3-8 Decoder | 4 | 3 | 3 |
|  | b | Explain 4bit Binary adder and Substractor | 4 | 3 | 2 |
| 8. | a | Design Mod-8 synchronous Counter? | 4 | 4 | 4 |
|  | b | with neat sketch Define the Operation of universal shift register | 4 | 4 | 2 |
| 9. | a | Design and explain the operation of Weighing machine | 6 | 5 | 3 |
|  | b | What are PLDs Classify them | 2 | 5 | 2 |

******