UNIT-V

INTRODUCTION TO DSP PROCESSORS

Common DSP Applications…

- Communications

- Audio, Video processing

- Graphics, 3-D rendering

- Navigation, radars, GPS

- Controls – Robotics, guidance, Machine Vision

- Filtering

- Frequency-Time transformations (FFT-IFFT)

Common DSP Tasks…

- Modulation-Demodulation, Error correction

- Noise reduction, equalization, echo cancellation

- Audio compression

- Vector and Matrix calculations

- Control algorithms

DSPs Need to Do…

- Efficient repetitive numerical calculations

- Maintain numeric fidelity

- Provide high memory bandwidth

  - Streaming data

- Real Time processing

DSPs Need to Minimize…

- Real Time execution unpredictability

- Memory use

- Power consumption

- Cost

- Development time

What Do DSPs Have?

- Specialized memory architecture (Harvard)

- Specialized parallel execution units

- Specialized addressing modes

- Specialized instruction sets for parallelexecution

- Specialized peripherals

Von Neumann Architecture

Harvard Architecture…

Harvard Architecture with Dual Data Memory

Summary:

- Von Neumann = Shares the same data bus and the same main memory for transfer storage of instructions and data of the programs.

- Von Neumann is better for desktop computers/high performance computers were cost to performance ratio is important.

- Harvard = Uses two separate buses for the transfer of data and instructions and two separate memories for storage of data and instructions.

- Harvard architecture is used primarily for small embedded computers and signal processing. Commonly used within CPUs to handle the cache.

Main differences

- Cell sizes used within the main memory are same in Von Neumann. However, Harvard allows for different cell sizes for data/instructions making effective use of resources.

- The programs in Harvard tend to be large.

- Modern computers make use of both Harvard and Von Neumann architecture.

- The main memory is used to store both instructions and data and they are both transferred over the data bus. However, the CPU’s cache has Harvard architecture.

- There is a separate cache memory for instructions and data.

- There is also a separate data bus between these caches.

Von Neumann Architecture

Advantages:

- Not only data but also instructions of programs are stored within the same memory.

- This makes it easier to re-program the memory.

- Less physical space is required than Harvard

- Handling just one memory block is simpler and easier to achieve

- Cheaper to use than Harvard

Disadvantages:

- Shared memory - a defective program can overwrite another in memory, causing it to crash

- Memory leaks - some defective programs fail to release memory when they are finished with it, which could cause the computer to crash due to insufficient memory

- Data bus speed - the CPU is much faster than the data bus, meaning it often sits idle (Von Neumann bottleneck)

- Fetch rate - data and instructions share the same data bus, even though the rate at which each needs to be fetched is often very different

- All memory cell sizes are the same and so can’t be different for instructions/data making it less efficient.

Harvard Architecture

Advantages :

- There is less chance of corruption since data and instructions are transferred via different buses.

- Data and instructions are accessed in the same way.

- Offers higher performance since Harvard allows for simultaneous fetching of data and instructions - they are kept in separate memory and travel via separate buses

- Both memories can use different cell sizes making effective use of resources.

- Greater memory bandwidth that is more predictable (separate memory for instructions and data)

Disadvantages:

- When there is free data memory it cannot be used for instructions and vice versa. Memory dedicated to each must be carefully balanced in manufacture.

- Production of a computer with two buses is more expensive and takes more time to manufacture.

- Harvard architecture has more pins so more complex for main board manufactures to implement.

- Not widely used so more difficult to implement

- Harvard architecture requires a control unit for two buses that is more complicated and development of which is expensive and needs more time.

VLIW (Very long instruction word)

- Very long instruction word (VLIW) describes a computer processing architecture  in which a language compiler or pre-processor breaks program instruction down into basic operations that can be performed by the processor in parallel (that is, at the same time).

- Multiple operations per instruction

- Compiler is in control

- Only RISC like operation support

  - Short cycle times

  - Easier to compile for

- Extensible / Scalable

- not binary compatible !!

What is RISC?

A reduced instruction set computer is a computer that only uses simple commands that can be divided into several instructions that achieve low-level operation within a single CLK cycle, as its name proposes “Reduced Instruction Set”.

RISC Architecture

The term RISC stands for ‘’Reduced Instruction Set Computer’’. It is a CPU design plan based on simple orders and acts fast.

- This is small or reduced set of instructions. Here, every instruction is expected to attain very small jobs.

- In this machine, the instruction sets are modest and simple, which help in comprising more complex commands.

- Each instruction is of a similar length; these are wound together to get compound tasks done in a single operation.

- Most commands are completed in one machine cycle.

- This pipelining is a crucial technique used to speed up RISC machines.

What is CISC?

A complex instruction set computer is a computer where single instructions can perform numerous low-level operations like a load from memory, an arithmetic operation, and a memory store or are accomplished by multi-step processes or addressing modes in single instructions, as its name proposes “Complex Instruction Set ”.

CISC Architecture

- The term CISC stands for ‘’Complex Instruction Set Computer’’.

- It is a CPU design plan based on single commands, which are skilled in executing multi-step operations.

- CISC computers have small programs.

- It has a huge number of compound instructions, which takes a long time to perform.

- Here, a single set of instruction is protected in several steps; each instruction set has additional than 300 separate instructions. Maximum instructions are finished in two to ten machine cycles.

- In CISC, instruction pipelining is not easily implemented.

Difference between RISC and CISC Architecture

| RISC | CISC |
| --- | --- |
| 1. RISC stands for Reduced Instruction Set Computer. | 1. CISC stands for Complex Instruction Set Computer. |
| 2. RISC processors have simple instructions taking about one clock cycle. The average clock cycle per instruction (CPI) is 1.5 | 2. CSIC processor has complex instructions that take up multiple clocks for execution. The average clock cycle per instruction (CPI) is in the range of 2 and 15. |
| 3. Performance is optimized with more focus on software | 3. Performance is optimized with more focus on hardware. |
| 4. It has no memory unit and uses separate hardware to implement instructions.. | 4. It has a memory unit to implement complex instructions. |
| 5. It has a hard-wired unit of programming. | 5. It has a microprogramming unit |
| 6. The instruction set is reduced i.e. it has only a few instructions in the instruction set. Many of these instructions are very primitive. | 6. The instruction set has a variety of different instructions that can be used for complex operations. |
| 7. The instruction set has a variety of different instructions that can be used for complex operations. | 7. CISC has many different addressing modes and can thus be used to represent higher-level programming language statements more efficiently. |
| 8. Complex addressing modes are synthesized using the software. | 8. CISC already supports complex addressing modes |
| 9. Multiple register sets are present | 9. Only has a single register set |
| 10. RISC processors are highly pipelined | 10. They are normally not pipelined or less pipelined |
| 11. The complexity of RISC lies with the compiler that executes the program | 11. The complexity lies in the microprogram |
| 12. Execution time is very less | 12. Execution time is very high |
| 13. Code expansion can be a problem | 13. Code expansion is not a problem |
| 14. The decoding of instructions is simple. | 14. Decoding of instructions is complex |
| 15. It does not require external memory for calculations | 15. It requires external memory for calculations |
| 16. The most common RISC microprocessors are Alpha, ARC, ARM, AVR, MIPS, PA-RISC, PIC, Power Architecture, and SPARC. | 16. Examples of CISC processors are the System/360, VAX, PDP-11, Motorola 68000 family, AMD, and Intel x86 CPUs. |
| 17. RISC architecture is used in high-end applications such as video processing, telecommunications and image processing. | 17. CISC architecture is used in low-end applications such as security systems, home automation, etc. |

TMS320C54x Internal Block Diagram

Architecture of C54x

- Fixed Point processor

- Advanced Harvard Architecture, CISC Processor – Separate memory bus structures for program & data.

- High degree of parallelism – Multiply, load/store, add/sub to/from ACC and new address generation can be done simultaneously.

- Powerful Instruction set & most of the operations are of single cycle

- Targeted for portable devices (cellular phones, MP3 players, digital cameras …)

Bus structure

Has several address/data buses:

1. Program Bus (PB): carries instruction codes & immediate operands from program memory to CPU.

2. Program Address Bus (PAB): provides addresses to program memory for both read/write operations.

3. Data Bus (DB): carries data between data memory space and CPU.

4. Data Address Bus (DAB): provides addresses to access data memory.

Buses in C54x

- 8 major 16-bit buses

- 4 program / data buses

1. Program bus, PB

2. Data buses

CB & DB for READ

EB for Write

- 4 address buses • PAB, CAB, DAB & EAB

- All CPU registers, peripheral registers and I/O ports occupy data memory space

Memory organization

- Minimum address range of 192K words

– 64K words for program space

– 64K words for data space

– 64K words for I/O space

- ROM, DARAM, SARAM, two way shared RAM

- On-chip Memory Security option

- MMR: 26 CPU regs, peripheral regs and scratch pad RAM block located on data page 0(DP0)

Central Processing Unit

- CPU Registers

- 40-bit ALU

- Two 40-bit Acc Regs (AccA & AccB)

- Barrel Shifter Supporting 0-31 bit left shift & 0-16 bit right shift range

- MAC Block

- 16-bit Temp Reg (T)

- 16-bit Transition Reg (TRN)

- Compare, Select and Store Unit (CSSU)

- Exponent Encoder

Accumulators A & B

CPU registers

- IMR, IFR

- ST0 & ST1

- PMST

- AR0 – AR7(GPRs)

- SP reg

- Circular-Buffer size Register (BK)

- Block-Rep Regs (BRC, RSA and REA)

- PC Extension Reg (XPC)

- ST0,ST1,PMST registers

Addressing Modes in TMS320C54X Processor

The method of specifying the operand or the data to be operated by the instruction.

- Immediate addressing

- Absolute addressing

- Accumulator addressing

- Direct addressing

- Indirect addressing

- Memory mapped register addressing

- Stack addressing

1) Immediate addressing mode:

- The data is specified as a part of the instruction.

- Value encoded in the instruction.

- Two types of values:

– Short immediate (3/5/8/9- bit constant)

–Long immediate (16 bits)

- # indicates immediate.

Example:	• LD #5, ARP; load the immediate 3-bit constant (5h)

• LD #143h, DP; load the immediate 9-bit constant (143h) in DP

• LD #80h, A; 8-bit constant

• LD #1000h, A; 16-bit constant

2) Absolute Addressing Mode:

- In this the 16-bit address of the operand is directly

- This addressing can be used to address an operand in all the three address spaces of the processor(i.e. address an operand in program memory, data memory and I/O ports)

- Complete address is specified

- Address is always of 16-bits

- 4 types:

– dmad addressing

– pmad addressing

– PA addressing

– *(lk) addressing

Example:

• MVKD SAMPLE,*AR5;	dmad addr

• MVDK *AR3, DATA1; 	dmad addr

• MVPD COEFF, *AR7; 	pmad addr

• MVPD COEFF, *AR7; 	pmad addr

• PORTR FIFO, *AR5; 		PA addr

• LD *(BUFFER), A; 		*(lk) addr

3) Accumulator Addressing Mode:

- In this the contents of accumulator is the address of the operand/data in program memory.

- Use Acc (A/B) contents as address.

- Used to address program memory as data.

- Two instructions:

– READA Smem

– WRITA Smem

4) Direct Addressing Mode:

- In this the lower 7 bits of data memory address are specified in the instruction itself.

- The 16-bit data memory address is formed by using either the 9 bits of DP(Data Pointer) in status register-0 or the 16-bit of SP(Stack Pointer)

- When DP is used, the 9 bits of DP is the upper 9 bits of the 16-bit address and the lower 7 bits are the address directly specified by the instruction.

- When SP is used, the 16-bit content of SP is added to 7 bits specified in the instruction to form 16-bit address.

Example: 	ADD 6ch, A; Add the content of memory directly addressed by the

instruction to Accumulator

SUB 57h, B;   Subtract the content of memory directly addressed by the

instruction to Accumulator B

5) Indirect addressing

- In this the data memory address is specified by the content of one of the eight auxiliary registers, i.e. AR0-AR7

- The AR(Auxiliary Register) currently used for accessing the data is denoted by 3-bit ARP(Auxiliary Register Pointer)

- In this addressing mode, the content of AR can be updated automatically either after or before the operand is fetched.

Example:  LD *AR3, A   ; load the content of memory addressed by AR3 in accumulator A

LD *AR3-, A    ; same as above, but after loading decrement AR3

LD *AR3+, A    ; same as above, but after loading increment AR3

LD *AR3-0, A    ; same as above, but after loading decrement AR3 using AR0

LD *AR3+0, A    ; same as above, but after loading increment  AR3 using AR0

6) Memory mapped register addressing

- In this the address of the memory-mapped register is specified as direct or indirect address in the instruction.

Example:  LDM 06h, A   ; Load the content of MMR directly addressed by the  instruction

in accumulator A

STLM A, 1Eh; Store the content of accumulator A in MMR directly addressed

by the instruction

7) Stack addressing

- In this the data memory address is the content of Stack Pointer(SP)

- The PUSH and POP instruction access the stack memory using  the stack addressing mode.

- The CALL interrupt and RETURN instructions also use stack pointer address for automatic storage/retrieval of information to/from stack.

Example: PSHM 1ch; Decrement SP by 2 and push the content of MMR addressed by

the instruction(address=1Ch)  to stack memory addressed by SP

POPM 1Ch; POP the top of stack pointed by SP to MMR addressed by the

instruction (address=1Ch), then SP in incremented by 2

INSTRUCTION SET OF TM320C54X PROCESSORS

- Arithmetic instructions

- Logical instructions

- Branch/control instructions

- Load/store instructions

- Move instructions

1) Arithmetic instructions

2) Logical instructions

3) Branch/control instructions

4) Load/store instructions

5) Move instructions